# Orbit Determination Methods and Propagation Techniques

Patrick Ramzy - rocketchat - patrick.ramsey@aero.org

Drew.Latta1 - rocketchat

June 3rd, 2025

​​Orbit Definition and Determination

- Semi major axis and inclination are key orbital parameters

- Ground stations track objects to know where to look at specific times and places around Earth

- Orbit Determination (OD) fits orbital solutions to observations

- Residuals and residual ratios check prediction accuracy vs actual position

- Measurements use sigma (standard deviation) for accuracy

### Output Formats

- Elset (TLE - Two Line Element):

  - Standard since 1930s

  - Sufficient accuracy to point telescopes

  - Contains Keplerian elements (angles, positions for reconstructing orbit)

  - Changes significantly after 3 days

- State Vectors:

  - Position + velocity for forward propagation

  - Can output covariance data

- Ephemeris:

  - Pre-computed set of state vectors

  - Used in satellite onboard software

  - Helpful when onboard orbit predictor is insufficient

  - Can be generated for 5-10 days forward using calibrated force models

### Propagation Methods

- Analytical Propagation:

  - Uses averaged motion models

  - More computationally efficient

  - Has assumptions baked into model

  - Less accurate over time

- Numerical Propagation:

  - Creates detailed force models

  - Uses sophisticated Newton’s method

  - Can easily add forces and accelerations

  - More computationally intensive and slower

  - Early 1970s example: Could process 10,000 objects on analytical propagator in same time as 3 objects on numerical propagator

### Solution Quality Evaluation

- RMS (Root Mean Square):

  - Average absolute value deviation

  - Helps quantify accuracy

- Covariance Analysis:

  - Derives from ephemeris or state vector

  - Models 95% probability ellipsoid around object

  - Quantifies multidimensional standard deviation of location

- Benchmarking Methods:

  - Compare against GPS ground truth when available

  - Run orbit determination on known objects to validate model

  - 500 meters accuracy is considered fairly good

  - Look at orbit fit residuals (15-year standard)

### Common Error Sources

- Cross-tags:

  - Leader-follower satellites (500m apart) can be mistaken as single object

  - Can detect by analyzing observation quality

- Miscalibration:

  - Atmospheric drag errors

  - Force model inaccuracies

- Coordinate Systems:

  - Must standardize between different systems

  - Errors from mixing heliocentric vs other coordinate systems

- UCTs (Uncorrelated Tracks):

  - Most common error is cross-tagging

  - Critical to maintain correct object identification

  - Solution quality degrades with mixed observations