Explanation

- The light intensity from space objects follows a Gaussian distribution

  - As you move away from the bright center, it gets dimmed out

  - This results in “smearing” the light

- Observatories often take measurements of this “smear”

  - For stellar object tracking (stars, satellites, etc), you want to minimize smear

  - This smear is referred to as Full Width Half Max (FWHM), measured in arc seconds

  - Higher FWHM measurements means more smear or blur in the light

- Visualization:

  - Smaller FWHM = Less smearing

  - Larger FWHM = More smearing

- To change FWHM into a more familiar unit of spreadiness, I converted FWHM into standard deviation, 𝜎

  - For the sake of making things easier to think about, let’s call this a “blur unit”

  - FWHM = 2.355𝜎

  - CAREFULLY NOTE: ALL THE PAPERS IN DOCUMENTATION USE FWHM, NOT STANDARD DEVIATION

- We will use 𝜎 to express how much uncertainty we have in elevation and azimuth measurements.

  - Gaussian distributions in spherical trig are a lot harder, but let us use the small angle approximation (angles on the order of arc seconds)

  - Example: We record

    - an azimuth of 40°

    - an elevation of 50°

    - a blur unit of 1 arcsecond

  - Then, we have this:

- Imagine the satellite is at 40° azimuth, 50° elevation. We have “an idea” of where it actually is

- Yellow/orange is higher probability

- Blue is lower probability

- We sample from this Gaussian

- The blur unit for a particular measurement is extremely dependent on atmospheric conditions and can vary widely

Results

- After examining FWHM data from various observatories, I have created the following:

  - A percentile chart

  - General heuristics

  - BE CAREFUL: ALL THESE MEASUREMENTS ARE IN ARCSECONDS, NOT DEGREES

    - 1° = 3600 arcseconds

Percentile Chart

- Below is a chart aggregating 518,092 measurements from Muztagh-ata Site II and Lijiang Observatories.

- This is not a complete representation of realistic measurements (you need weather data from everywhere), but it is a good start

- Referencing the below chart:

  - 25th Accumulation: A blur unit of 0.30 arc seconds is worse than 25% of all measurements; better than 75% of all measurements

  - 50th Accumulation: A blur unit of 0.375 arc seconds is “typical” (median)

  - 75th Accumulation: A blur unit of 0.4875 arc seconds is worse than 75% of all measurements; better than 25% of all measurements

    - On a “bad day”, the blur unit is 0.4875 arc seconds

General Heuristics

- It appears that the “best reasonable” blur unit is 0.25 arcseconds

  - Context: Taken at Maidanek Observatory

- Observations at Cello Chico, Cello Toco sites in Atacama, Chile suggests that about 0.28 to 0.40 arcseconds are easily doable

- I found that the worst recorded blur unit is 1.32 arcseconds

  - Context: Taken at Antarctic stations

- Some Figures To Run With

  - Good Day: A blur unit of ~0.3000 arc seconds (better than 75% of days)

  - An OK Day: A blur unit of ~0.3750 arc seconds (better than 50% of days)

  - A Bad Day: A blur unit of ~0.4875 arc seconds (worse than 75% of days)

  - A REALLY Bad Day: A blur unit of ~0.7000 arc seconds (worse than 90% of days)

- Results taken from Antarctic Stations, Atacama, Lijiang, Muztagh-ata, Maidenak, and Xinglong all show that the mean blur unit is much worse than the median blur unit

  - Essentially, most days are good

  - But when it gets bad, it gets bad

  - The FWHM distribution for Lijiang is shown below. It is noticeably skew right

Documentation

- Measurement of optical seeing on the high antarctic plateau

- Simultaneous seeing measurements at Atacama

- Astronomical Site Monitoring System at Lijiang Observatory

- Site testing at Muztagh-ata site II: seeing statistics

- Astronomical Seeing and Meteorological Parameters at Maidanak Observatory

- Astronomical Observing Conditions at Xinglong Observatory from 2007 to 2014