# Real-world (non-synthetic) UCT / Unassociated Object Benchmark Datasets & Eval Approaches

PLEASE READ:

This doc contains the two clearest publicly documented cases where researchers bench-marked more than one UCT/association algorithm on the same set of real satellite measurements (i.e., no synthetic data).

In the description of both papers, i've include:

- Algorithms Tested on Real-World Data

- Real-World Objects (Satellites Used)

- Test Cases (“Real Testing Scenarios”)

- Results and “Grading” of Algorithm Performance

- Summary of “Grading” per Algorithm (All Maneuvers Combined)

- Key Takeaways

## Montilla et al., “Manoeuvre detection in Low Earth Orbit with Radar Data” (Advances in Space Research, 2022)

#### 1. Algorithms Tested on Real-World Data

Four algorithms were benchmarked on real satellite data:

- UKF (Unscented Kalman Filter)

- Algorithm 1 (reachability-based maneuver detection)

- Algorithm 2 (extended reachability metrics)

- MDF (Manoeuvre Detection Filter – a hybrid UKF + Algorithm 1 design)

#### 2. Real-World Objects (Satellites Used)

The following seven satellites provided the real orbital data for all test cases:

- Sentinel-1A

- Sentinel-1B

- Sentinel-2A

- Sentinel-2B

- Swarm-C

- TanDEM-X

- TerraSAR-X

#### 3. Test Cases (“Real Testing Scenarios”)

Fifteen distinct scenarios (numbered 1–15) were defined, each covering a continuous observation window split into segments between consecutive radar tracks. Each segment was processed individually (with initialization from precise orbits), while each filter ran continuously across the full scenario. Key attributes of these scenarios are summarized in Table 4:

| Scenario # | Epoch of Most Intense Maneuver | Observation Window (UTC) | Total Segments | Segments with Maneuvers |
| --- | --- | --- | --- | --- |
| 1 | 14 Aug 2019 23:11:03 | 06 Aug 2019 06:52:43 → 16 Aug 2019 18:09:54 | 12 | 2 |
| 2 | 29 Jan 2020 23:11:08 | 23 Jan 2020 06:36:14 → 31 Jan 2020 07:09:05 | 8 | 1 |
| 3 | 21 May 2020 01:06:08 | 17 May 2020 06:27:59 → 25 May 2020 18:01:43 | 10 | 1 |
| 4 | 19 Aug 2020 22:29:28 | 19 Aug 2020 17:45:21 → 27 Aug 2020 18:18:18 | 9 | 2 |
| 5 | 14 Aug 2019 23:59:53 | 05 Aug 2019 07:00:17 → 17 Aug 2019 07:00:15 | 13 | 2 |
| 6 | 19 Sep 2019 00:57:21 | 10 Sep 2019 18:00:59 → 22 Sep 2019 07:00:19 | 12 | 1 |
| 7 | 20 May 2020 22:37:36 | 12 May 2020 18:09:12 → 28 May 2020 17:36:30 | 17 | 1 |
| 8 | 17 Jun 2020 22:05:17 | 09 Jun 2020 17:36:29 → 24 Jun 2020 18:00:59 | 18 | 1 |
| 9 | 05 Feb 2020 16:09:00 | 03 Feb 2020 22:25:25 → 13 Feb 2020 22:25:22 | 8 | 1 |
| 10 | 10 Sep 2020 17:10:27 | 03 Sep 2020 11:22:01 → 10 Sep 2020 22:25:35 | 6 | 1 |
| 11 | 18 Sep 2019 16:59:55 | 11 Sep 2019 11:01:44 → 21 Sep 2019 22:25:30 | 5 | 1 |
| 12 | 17 Sep 2020 16:10:04 | 08 Sep 2020 11:22:02 → 19 Sep 2020 10:51:42 | 9 | 1 |
| 13 | 15 Jul 2020 17:02:15 | 06 Jul 2020 01:47:22 → 20 Jul 2020 00:05:35 | 12 | 1 |
| 14 | 22 Aug 2020 00:24:06 | 17 Aug 2020 17:49:25 → 24 Aug 2020 06:48:33 | 4 | 4 |
| 15 | 22 Aug 2020 00:24:07 | 16 Aug 2020 18:06:50 → 22 Aug 2020 17:58:07 | 4 | 4 |

Table 4. List of real testing scenarios. A segment spans from one radar track to the next; segments marked “with maneuvers” contain one or more actual orbit corrections.

#### 4. Results and “Grading” of Algorithm Performance

Each algorithm was evaluated on its ability to detect the hidden maneuvers in the 15 scenarios (134 total segments without maneuvers, plus segments containing known maneuvers). Performance was measured by three core metrics overall (across all scenarios):

- % Maneuvers Detected

- % False Positives (declaring a maneuver when none occurred)

- % False Negatives (failing to detect a true maneuver)

Below is a concise summary (all scenarios combined):

- UKF

  - Detected 41.66% of all true maneuvers

  - False Positives ≈ 2.98% (i.e., very few spurious detections)

  - False Negatives ≈ 58.34% (i.e., missed the majority of maneuvers)

  - Note: Per‐scenario grading shows UKF struggled most when segments were long or data were sparse (e.g., TanDEM-X/TerraSAR-X high-maneuver cases).

- Algorithm 1 (reachability-based)

  - Detected 54.16% of all true maneuvers

  - False Positives ≈ 45.84% (many spurious alerts, especially in short segments with few measurements)

  - False Negatives ≈ 8.21% (missed very few maneuvers in segments where it triggered)

  - Note: It performed well (83.3%–100% maneuver detection) on Scenarios 1–4 but saw degraded detection and more false positives in Scenarios 5–10.

- Algorithm 2 (extended reachability metrics, e.g., P1M)

  - P1M detected 54.16% of true maneuvers (same aggregate as Algorithm 1 in best cases)

  - False Positives ≈ 45.84% (comparable to Algorithm 1 overall)

  - False Negatives ≈ 8.21% (similar low miss‐rate in early scenarios but varied in others)

  - Note: Algorithm 2 outperformed Algorithm 1 specifically in TanDEM-X/TerraSAR-X (Scenarios 14–15), detecting 75% of maneuvers there versus Algorithm 1’s 50%.

- MDF (Manoeuvre Detection Filter – UKF + Algorithm 1 hybrid)

  - Achieved higher detection rates than any single method, especially in high-maneuver scenarios:

    - E.g., in Scenarios 14–15 (four maneuvers each), 75%–100% of maneuvers were detected, whereas UKF/Algorithm 1 alone detected 25%–50%.

  - False Negatives were significantly reduced by dynamically inflating covariance after each detection, allowing the filter to recover quickly.

  - False Positives remained low compared to Algorithm 1/2, thanks to the UKF backbone rejecting improbable spikes.

  - Example: In a TanDEM-X maneuver (Figure 15), MDF detected the maneuver immediately and quickly recovered, whereas UKF alone missed it.

#### 5. Summary of “Grading” per Algorithm (All Maneuvers Combined)

| Algorithm | % Detected | % False Positives | % False Negatives | Notes on Tracking “Known but Unknown” Objects |
| --- | --- | --- | --- | --- |
| UKF | 41.66 % | 2.98 % | 58.34 % | Good at avoiding false alarms, but missed most maneuvers—struggled when maneuvers occurred in long propagation gaps. |
| Alg 1 | 54.16 % | 45.84 % | 8.21 % | High detection in early scenarios (1–4); many false alarms in segments with sparse data; low miss-rate when it did trigger. |
| Alg 2 | 54.16 %* | 45.84 %* | 8.21 %* | Similar aggregate to Alg 1; however, it outperformed Alg 1 in dense-maneuver cases (Scenarios 14–15), detecting 75 % of maneuvers there. |
| MDF (UKF+1) | ≈ 62 %–75 %† | < 5 % | < 38 % | Combined the robust orbit propagation of UKF with Alg 1’s reachability checks. Improved detection (especially in high-maneuver scenarios) and reduced false negatives substantially. |

*Aggregate P1M results mirrored Alg 1 in Scenarios 1–4; variations occur in later groups.
 †Exact overall percentages vary by metric (P1M vs. P1D) and scenario group; in high-maneuver scenarios (14–15), MDF detected 75 %–100 % of maneuvers.

#### 6. Key Takeaways

- Four algorithms were evaluated on seven real satellites across 15 multi-segment scenarios (Table 4).

- UKF alone tracked only ~41.7 % of hidden maneuvers but maintained very low false positives.

- Reachability-based methods (Alg 1/Alg 2) raised detection to ~54 % but incurred many false alarms (∼46 %).

- MDF (UKF + Alg 1) balanced both: it detected most maneuvers—especially in dense-maneuver cases—while keeping false positives low, effectively tracking “known” satellites under the guise of “unknown” objects.

This concise breakdown reflects how each method fared when forced to “rediscover” a known orbit while masking it as unknown, grading them by maneuver detection, false alerts, and missed maneuvers.

## Miller, “Special Perturbations Uncorrelated Track Processing” (AAS 07-228, MITRE, 2007)

#### 1. Algorithms Tested on the “Stressing” Real-World Test Case

Four UCT‐processing methods were evaluated on the same set of withheld (hidden) tracks from three closely spaced debris objects. Two are legacy GP-based approaches (as baselines), and two are new SP-based algorithms:

- Cuthbert–Morris (C-M) (General Perturbations clustering)

- Lockheed Martin Pattern Recognition Tool (GP element-set trend‐analysis)

- SP Hierarchical Agglomerative Clustering (complete-link variant with SP differential-correction RMS proximity)

- SP Track-Oriented Multiple Hypothesis Tracking (MHT) (using SP DC RMS for gating)

#### 2. Real-World Objects (Satellites) Used

- Three small debris objects (“objects 1, 2, 3”) whose orbits are nearly identical in GNSS‐derived element sets; all three were originally tracked as UCTs before cataloging.

- No commercial or military “names” are given—each debris object is identified only by its “object #” in the paper.

#### 3. Stressing Test Case Overview

- Objective: Force each algorithm to “rediscover” which tracks belong to which of the three unknown objects, despite sparse measurements and near-identical orbit parameters.

- Data:

  - Nine separate one-track element sets (UCTs), each comprised of three observations spaced ~10 seconds apart.

  - Tracks were derived over a window of days (days 212–222 of 2006) using a sliding GP differential-correction interval (Figure 1 & 2).

  - To simulate uncorrelated tracks, each track’s true satellite label was replaced by a dummy “9xxxx” tag, hiding its identity.

- Element-Set Characteristics:

  - GP Operational Sets (Figure 2): Produced by batch GP DC across multiple tracks → linear trends visible, orbits distinguishable by period/inclination.

  - GP One-Track Sets (Figure 3): Produced by single-track initial orbit determination (Herrick–Gibbs) + SGP4 DC → large estimation errors, trends no longer separable by object.

  - Test UCTs: The nine one-track GP element sets (Figure 3) serve as inputs; algorithms must cluster/associate them correctly to reveal “object 1 vs 2 vs 3.”

#### 4. How Each Algorithm Was Evaluated

- Cuthbert–Morris (GP)

  - Clusters UCTs by matching orbital plane, node‐drift rate, and orbital period.

  - Result: Merged all nine tracks into a single cluster corresponding to object 2; failed to separate the three objects.

- Lockheed Martin Pattern Recognition (GP)

  - Supervised ART pattern recognition on plots of GP element-set parameters over time.

  - Result: Produced no clusters (zero “hits”); could not discriminate any of the three debris objects.

- SP Hierarchical Agglomerative Clustering

  - Pattern Representation: SP state vectors + covariance from one-track SP DC (Herrick–Gibbs → SP DC).

  - Proximity Measures Tested:

    - (a) Eq (2): $d^2(x_i,x_j);=;(r_j - r_i)^T(R_i + R_j)^{-1}(r_j - r_i);+;(v_j - v_i)^T(V_i + V_j)^{-1}(v_j - v_i)$
 – Where $r,;v$ are position/velocity, $R,;V$ their respective 3×3 covariances.

    - (b) Eq (3): RMS of SP DC fit when using both tracks together (always positive).

  - Clustering Methodologies:

    - Single-Link (SL): Minimizes minimum inter-cluster distance → tends to “chaining” errors.

    - Complete-Link (CL): Minimizes maximum inter-cluster distance → forms tighter, more spherical clusters.

  - Results (Using Nine UCTs):

    - SL + Eq (2) or Eq (3):
 • Produced erroneous clusters—no natural “cut” that yields three groups correctly (Tables 1 & 4).
 • Early mistakes in bottom-up merges propagated upward (Figures 8 & 10).

    - CL + Eq (2):
 • Clustering still failed to yield correct “three clusters” (Table 2 & Figure 9): some tracks from different objects ended up merged.

    - CL + Eq (3) (RMS Proximity):
 • Correctly recovered three clusters (Table 5 & Figure 11):
 – Cluster 1: Tracks {1, 2, 3} → Object 1
 – Cluster 2: Tracks {4, 5, 6} → Object 2
 – Cluster 3: Tracks {7, 8, 9} → Object 3
 • The RMS criterion effectively distinguishes actual match-ups by penalizing poor two-track fits.

- SP Track-Oriented MHT

  - Approach: Builds a global tree of all possible track-to-track associations over nine UCTs; prunes unlikely hypotheses based on SP DC RMS.

  - Gating:

    - Use Eq (3) RMS; threshold = 3.0 (not to exclude any true match).

    - Some “false” associations allowed initially (e.g., (1, 5), (1, 9), etc.), but ultimately pruned.

  - Hypothesis Tree Construction (Figure 12):

    - UCT insertion order: 4 → 1 → 7 → 5 → 6 → 2 → 3 → 8 → 9

    - At each new UCT, branch into all existing trees where RMS < 3.0 → yields seven root tracks (4, 1, 7, 5, 2, 3, 8).

  - Global Pruning:

    - Discard leaves at depth < 3 (insufficient tracks to form a valid SP DC).

    - For depth ≥ 3, compute SP DC RMS over first three UCTs; remove any leaf whose RMS > RMS threshold in Table 6.

    - Survivors (correct triplets):

      - {4, 5, 6} → Object 2 (RMS = 1.5575)

      - {1, 2, 3} → Object 1 (RMS = 1.9562)

      - {7, 8, 9} → Object 3 (RMS = 1.5503)

    - All other partial hypotheses (e.g., {1, 5, 6} with RMS = 102.45, etc.) are pruned.

#### 5. Performance “Grading” (Did Each Method Reconstruct All Three Objects?)

| Method | Correct “3-Cluster” Outcome? | Comments |
| --- | --- | --- |
| GP Cuthbert–Morris (C-M) | No | All nine tracks collapsed into one cluster (object 2) → Could not separate objects 1 and 3. |
| GP Lockheed Martin Pattern Recognition | No | Generated zero clusters → completely failed to detect any grouping. |
| SP Hierarchical SL (Eq 2 or Eq 3) | No | “Chaining” errors: single-link merges different objects early → incorrect 3-cluster cuts (Tables 1 & 4). |
| SP Hierarchical CL (Eq 2) | No | Merged tracks {3, 4} erroneously → propagated error → never yielded three correct clusters (Table 2). |
| SP Hierarchical CL (Eq 3, RMS proximity) | Yes | Yielded exactly three clusters matching true debris objects (Table 5 & Figure 11). Clusters: {1, 2, 3}, {4, 5, 6}, {7, 8, 9}. |
| SP Track-Oriented MHT (RMS gating + pruning) | Yes | Correctly associated all tracks into three object trees: {1, 2, 3}, {4, 5, 6}, {7, 8, 9}. All false hypotheses pruned via SP DC RMS (Table 6 & Figure 12). |

#### 6. Key Takeaways

- Legacy GP Techniques (C-M & LM) fail entirely when element-set errors mask orbital differences—no valid clusters emerge.

- SP Hierarchical Clustering only succeeds if:

  - (a) Complete-Link linkage (avoids SL’s “chaining”), and

  - (b) RMS Proximity (Eq 3) rather than covariance-based distance (Eq 2).

- SP MHT recovers correct groupings by maintaining multiple association hypotheses and pruning via RMS of three-track SP DC fits.

- Result: Both SP CL(Eq 3) and SP MHT “track” each known object as if it were unknown, correctly grading all nine UCTs into three real satellites—whereas GP methods fail under sparse, noisy, near-coorbital conditions.